GRE 隧道架构与配置文档

GRE Tunnel Architecture and Deployment Guide

# 1. 架构概览

## 1.1 四台服务器

London区（eu-west-2）vpc-031855222bf9628c8 (AVC-LONDON_172.18.0.0)

London-5 EC2

Tokyo区（ap-northeast-1）vpc-0efab2a1f077e427f (AVC-TOKYO_172.16.0.0)

Tokyo-2 EC2、Tokyo-3 EC2

Frankfurt区（eu-central-1）vpc-088786b0a27203924 (AVC-FRANKFURT_172.17.0.0)

Frankfurt-1 EC2

## 1.2 网络连接

London 与 Tokyo 通过 london<->tokyo海底光缆（Avalocom服务）互通

Tokyo 与 Frankfurt 通过 frankfurt<->tokyo海底光缆（Avalocom服务）互通

## 1.3 ip

|  | public ip (internet) | private ip (aws) |  | gre1 (10.0.1.0) | gre3 (10.0.3.0) |  | gre2 (10.0.2.0) | gre4 (10.0.4.0) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| London-5 | 52.56.77.3 | 172.18.0.147 | 海底光缆 | 10.0.1.1 | 10.0.3.1 |  |  |  |
| Tokyo-2 | 52.196.98.153 | 172.16.9.209 |  | 10.0.1.2 |  | 海底光缆 | 10.0.2.2 |  |
| Tokyo-3 | 13.159.217.158 | 172.16.8.27 |  |  | 10.0.3.2 |  |  | 10.0.4.2 |
| Frankfurt-1 | 63.181.101.84 | 172.17.10.112 |  |  |  |  | 10.0.2.1 | 10.0.4.1 |

# 2.  配置 GRE 隧道

## 2.1 配置

每台ec2上的每个gre隧道需要一个setup脚本和一个开机运行之的服务。

London-5:

/usr/local/bin/setup_gre1.sh

/etc/systemd/system/gre1-setup.service

/usr/local/bin/setup_gre3.sh

/etc/systemd/system/gre3-setup.service

Tokyo-2：

/usr/local/bin/setup_gre1.sh

/etc/systemd/system/gre1-setup.service

/usr/local/bin/setup_gre2.sh

/etc/systemd/system/gre2-setup.service

Tokyo-3：

/usr/local/bin/setup_gre3.sh

/etc/systemd/system/gre3-setup.service

/usr/local/bin/setup_gre4.sh

/etc/systemd/system/gre4-setup.service

Frankfurt-1

/usr/local/bin/setup_gre2.sh

/etc/systemd/system/gre2-setup.service

/usr/local/bin/setup_gre4.sh

/etc/systemd/system/gre4-setup.service

## 2.2 Tokyo-2 配置示例

/usr/local/bin/setup_gre1.sh（需要x权限）

Bash #!/usr/bin/env bash set -euo pipefail ####################################### # Configuration ####################################### # Tunnel TUN_NAME="gre1" TUN_MODE="gre" TTL=255 MTU=1400 # Underlay (public / VPC IPs) LOCAL_IP="172.16.9.209" REMOTE_IP="172.18.0.147" # Overlay (GRE point-to-point subnet) TUN_SUBNET="10.0.1.0/30" TUN_IP="10.0.1.2/30" # Outgoing interface for NAT EGRESS_IFACE="ens5" ####################################### # Sanity checks ####################################### command -v ip >/dev/null ¦¦ { echo "ip command not found"; exit 1; } command -v iptables >/dev/null ¦¦ { echo "iptables command not found"; exit 1; } ####################################### # Cleanup previous tunnel (idempotent) ####################################### ip tunnel del "$TUN_NAME" 2>/dev/null ¦¦ true ####################################### # Create GRE tunnel ####################################### ip tunnel add "$TUN_NAME" \ mode "$TUN_MODE" \ local "$LOCAL_IP" \ remote "$REMOTE_IP" \ ttl "$TTL" ip addr add "$TUN_IP" dev "$TUN_NAME" ip link set "$TUN_NAME" mtu "$MTU" up echo "GRE tunnel $TUN_NAME created: $LOCAL_IP ↔ $REMOTE_IP ($TUN_IP)" ####################################### # NAT for GRE overlay subnet ####################################### if ! iptables -t nat -C POSTROUTING -s "$TUN_SUBNET" -o "$EGRESS_IFACE" -j MASQUERADE 2>/dev/null; then iptables -t nat -A POSTROUTING -s "$TUN_SUBNET" -o "$EGRESS_IFACE" -j MASQUERADE echo "NAT rule added for $TUN_SUBNET via $EGRESS_IFACE" else echo "NAT rule already exists for $TUN_SUBNET" fi ####################################### # Done ####################################### echo "GRE setup complete." exit 0

/etc/systemd/system/gre1-setup.service

Plain Text [Unit] Description=GRE tunnel setup for relay Wants=network-online.target After=network-online.target ConditionPathExists=/usr/local/bin/setup_gre1.sh [Service] Type=oneshot # If your script isn't already root-only, ensure it runs as root (default for system unit is root anyway). ExecStart=/usr/local/bin/setup_gre1.sh # Useful if the script is idempotent and you want to re-run it on start/restart. RemainAfterExit=yes # Retry behavior (optional but nice) Restart=on-failure RestartSec=3 # Logging: send stdout/stderr into journald with a stable tag StandardOutput=journal StandardError=journal SyslogIdentifier=setup-gre1 # If your script relies on PATH differences under systemd, define a safe PATH Environment="PATH=/usr/sbin:/usr/bin:/sbin:/bin" [Install] WantedBy=multi-user.target

## 2.3 查看本机的所有gre通道

Bash (base) charles@tokyo-2:~$ ip tunnel show gre0: gre/ip remote any local any ttl inherit nopmtudisc gre1: gre/ip remote 172.18.15.102 local 172.16.9.209 ttl 255

## 2.4 查看gre1通道的ip

SQL (base) charles@tokyo-2:~$ ip addr show gre1 6: gre1@NONE: <POINTOPOINT,NOARP,UP,LOWER_UP> mtu 1400 qdisc noqueue state UNKNOWN group default qlen 1000 link/gre 172.16.9.209 peer 172.18.15.102 inet 10.0.0.2/30 scope global gre1 valid_lft forever preferred_lft forever inet6 fe80::5efe:ac10:9d1/64 scope link valid_lft forever preferred_lft forever

# 3 对特定域名的流量启用GRE路由

## 3.1 配置

每台ec2的每个gre隧道需要一个ip更新脚本，一个运行服务，和一个定时器

London-5:

/usr/local/bin/update_gre1_routes.sh

/etc/systemd/system/update-gre1-routes.service

/etc/systemd/system/update-gre1-routes.timer

/usr/local/bin/update_gre3_routes.sh

/etc/systemd/system/update-gre3-routes.service

/etc/systemd/system/update-gre3-routes.timer

Tokyo-2：

/usr/local/bin/update_gre1_routes.sh

/etc/systemd/system/update-gre1-routes.service

/etc/systemd/system/update-gre1-routes.timer

/usr/local/bin/update_gre2_routes.sh

/etc/systemd/system/update-gre2-routes.service

/etc/systemd/system/update-gre2-routes.timer

Tokyo-3：

/usr/local/bin/update_gre3_routes.sh

/etc/systemd/system/update-gre3-routes.service

/etc/systemd/system/update-gre3-routes.timer

/usr/local/bin/update_gre4_routes.sh

/etc/systemd/system/update-gre4-routes.service

/etc/systemd/system/update-gre4-routes.timer

Frankfurt-1

/usr/local/bin/update_gre2_routes.sh

/etc/systemd/system/update-gre2-routes.service

/etc/systemd/system/update-gre2-routes.timer

/usr/local/bin/update_gre4_routes.sh

/etc/systemd/system/update-gre4-routes.service

/etc/systemd/system/update-gre4-routes.timer

## 3.2 Tokyo-2 配置示例

/usr/local/bin/update_gre1_routes.sh（需要x权限）

Bash #!/usr/bin/env bash set -euo pipefail ####################################### # Config ####################################### HOSTNAMES=( api.kraken.com ws.kraken.com vip-ws.kraken.com eu-west-1-2.vip-ws.kraken.com ws-auth.kraken.com vip-ws-auth.kraken.com eu-west-1-2.vip-ws-auth.kraken.com ) INTERFACE="gre1" GATEWAY="10.0.1.1" # Optional: force DNS resolver to avoid local hijack (leave empty to use system default) DNS_SERVER="" # e.g. "1.1.1.1" or "8.8.8.8" # Safety: never add routes for these ranges DROP_LOOPBACK=true # drop 127.0.0.0/8 DROP_RFC1918=false # set true if you also want to drop 10/8,172.16/12,192.168/16 ####################################### # Helpers ####################################### die() { echo "ERROR: $*" >&2; exit 1; } require_cmd() { command -v "$1" >/dev/null 2>&1 ¦¦ die "Missing command: $1"; } is_ipv4() { [[ "$1" =~ ^([0-9]{1,3}\.){3}[0-9]{1,3}$ ]] } is_loopback() { [[ "$1" =~ ^127\. ]]; } is_rfc1918() { [[ "$1" =~ ^10\. ]] ¦¦ \ [[ "$1" =~ ^192\.168\. ]] ¦¦ \ [[ "$1" =~ ^172\.(1[6-9]¦2[0-9]¦3[0-1])\. ]] } resolve_ips() { local host="$1" if [[ -n "$DNS_SERVER" ]]; then dig +short "$host" @"$DNS_SERVER" else dig +short "$host" fi ¦ grep -Eo '([0-9]{1,3}\.){3}[0-9]{1,3}' ¦ sort -u } already_routed() { local ip="$1" ip route get "$ip" 2>/dev/null ¦ grep -q "via $GATEWAY dev $INTERFACE" } ####################################### # Main ####################################### require_cmd dig require_cmd ip # Must be root (don’t mix sudo inside the script) if [[ "${EUID:-$(id -u)}" -ne 0 ]]; then die "Run as root: sudo $0" fi # Basic validation ip link show "$INTERFACE" >/dev/null 2>&1 ¦¦ die "Interface not found: $INTERFACE" echo "Routing selected hostnames via $INTERFACE (gateway $GATEWAY)" [[ -n "$DNS_SERVER" ]] && echo "Using DNS server: $DNS_SERVER" for host in "${HOSTNAMES[@]}"; do mapfile -t ips < <(resolve_ips "$host" ¦¦ true) if [[ "${#ips[@]}" -eq 0 ]]; then echo "WARN: no A records for $host (skipped)" continue fi added=() skipped=() dropped=() for ipaddr in "${ips[@]}"; do # extra safety checks if ! is_ipv4 "$ipaddr"; then dropped+=("$ipaddr") continue fi if $DROP_LOOPBACK && is_loopback "$ipaddr"; then echo "WARN: $host resolved to loopback ($ipaddr). DNS/hosts problem? (skipped)" dropped+=("$ipaddr") continue fi if $DROP_RFC1918 && is_rfc1918 "$ipaddr"; then dropped+=("$ipaddr") continue fi if already_routed "$ipaddr"; then skipped+=("$ipaddr") else ip route replace "$ipaddr" via "$GATEWAY" dev "$INTERFACE" added+=("$ipaddr") fi done [[ "${#added[@]}" -gt 0 ]] && echo "Added $host: ${added[*]}" [[ "${#skipped[@]}" -gt 0 ]] && echo "Skipped $host: ${skipped[*]}" [[ "${#dropped[@]}" -gt 0 ]] && echo "Dropped $host: ${dropped[*]}" done exit 0

/etc/systemd/system/update-gre1-routes.service

Plain Text [Unit] Description=Update IP routes via GRE Wants=network-online.target After=network-online.target ConditionPathExists=/usr/local/bin/update_gre1_routes.sh [Service] Type=oneshot # Ensure predictable environment Environment="PATH=/usr/sbin:/usr/bin:/sbin:/bin" # Optional: wait until GRE interface exists ExecStartPre=/usr/sbin/ip link show gre1 ExecStart=/usr/local/bin/update_gre1_routes.sh # Allow manual re-run without stopping anything RemainAfterExit=yes # Retry if DNS / network not ready yet Restart=on-failure RestartSec=5 # Better logging StandardOutput=journal StandardError=journal SyslogIdentifier=update-gre1-routes [Install] WantedBy=multi-user.target

/etc/systemd/system/update-gre1-routes.timer

Plain Text [Unit] Description=Run GRE route updater every minute [Timer] # First run shortly after boot OnBootSec=30s # Re-run every minute after last successful run OnUnitActiveSec=60s # Avoid stampede if many timers exist AccuracySec=10s # Ensure missed runs execute after downtime Persistent=true Unit=update-gre1-routes.service [Install] WantedBy=timers.target

## 3.3 查看某域名的ip

Bash (base) charles@tokyo-2:~$ dig +short ws.kraken.com 104.17.188.205 104.17.189.205 104.17.185.205 104.17.186.205 104.17.187.205

## 3.4 查看某ip是否在gre1的路由表中

Bash (base) charles@tokyo-2:~$ ip route show dev gre1 3.10.184.172 via 10.0.0.1 10.0.0.0/30 proto kernel scope link src 10.0.0.2 13.42.167.97 via 10.0.0.1 13.43.106.133 via 10.0.0.1 18.202.121.20 via 10.0.0.1 52.31.171.196 via 10.0.0.1 54.73.136.16 via 10.0.0.1 104.17.185.205 via 10.0.0.1 104.17.186.205 via 10.0.0.1 104.17.187.205 via 10.0.0.1 104.17.188.205 via 10.0.0.1 104.17.189.205 via 10.0.0.1

## 3.5 查看去往某域名的路由

Bash (base) charles@tokyo-2:~$ traceroute ws.kraken.com traceroute to ws.kraken.com (104.17.187.205), 30 hops max, 60 byte packets 1 10.0.1.1 (10.0.1.1) 140.610 ms 140.060 ms 140.633 ms 2 ec2-52-56-0-85.eu-west-2.compute.amazonaws.com (52.56.0.85) 142.564 ms ec2-52-56-0-95.eu-west-2.compute.amazonaws.com (52.56.0.95) 141.550 ms ec2-52-56-0-81.eu-west-2.compute.amazonaws.com (52.56.0.81) 141.992 ms 3 100.65.18.144 (100.65.18.144) 168.908 ms 100.65.16.16 (100.65.16.16) 148.518 ms 100.65.19.160 (100.65.19.160) 143.268 ms 4 100.66.8.132 (100.66.8.132) 147.205 ms 100.66.8.232 (100.66.8.232) 144.829 ms 100.66.8.54 (100.66.8.54) 144.556 ms 5 100.66.11.2 (100.66.11.2) 143.871 ms 100.66.11.64 (100.66.11.64) 141.709 ms 100.66.11.200 (100.66.11.200) 147.867 ms 6 240.2.156.5 (240.2.156.5) 140.796 ms 240.2.156.4 (240.2.156.4) 140.359 ms 240.2.156.6 (240.2.156.6) 140.454 ms 7 * * * 8 240.2.112.38 (240.2.112.38) 141.122 ms * * 9 * 242.6.29.1 (242.6.29.1) 200.698 ms * 10 * * * 11 * 99.83.89.19 (99.83.89.19) 141.824 ms 150.664 ms 12 141.101.71.135 (141.101.71.135) 141.843 ms 141.101.71.93 (141.101.71.93) 141.864 ms 141.101.71.5 (141.101.71.5) 143.021 ms 13 104.17.187.205 (104.17.187.205) 141.741 ms 141.656 ms 141.494 ms

# 4 防火墙（UFW）配置（必须启用）

## 4.1 通用配置

允许 22/80/443 入站

允许icmp

允许gre

默认拒绝其余流量

## 4.2 Tokyo-2配置示例

一键脚本

Bash #!/usr/bin/env bash set -euo pipefail EGRESS=ens5 GRES=(gre1 gre2) [[ $(id -u) -eq 0 ]] ¦¦ { echo "Run: sudo $0"; exit 1; } # sanity ip link show "$EGRESS" >/dev/null for g in "${GRES[@]}"; do ip link show "$g" >/dev/null; done # disable ipv6 in ufw (avoid v6 noise) sed -i 's/^IPV6=.*/IPV6=no/' /etc/default/ufw # ---- allow ICMP via ufw before.rules (works on all ufw versions) ---- BR=/etc/ufw/before.rules cp -a "$BR" "${BR}.bak.$(date +%Y%m%d_%H%M%S)" # Insert ICMP accept rules once (idempotent) if ! grep -q "## allow-icmp" "$BR"; then awk ' BEGIN{ins=0} { print $0 if (!ins && $0 ~ /^\*filter$/) { print "## allow-icmp" print "-A ufw-before-input -p icmp -j ACCEPT" print "-A ufw-before-forward -p icmp -j ACCEPT" ins=1 } } ' "$BR" > "${BR}.tmp" && mv "${BR}.tmp" "$BR" fi # clean rebuild ufw --force disable ufw --force reset ufw default deny incoming ufw default allow outgoing ufw default allow routed # inbound services ufw allow 22/tcp ufw allow 80/tcp ufw allow 443/tcp ufw allow 53/tcp ufw allow 53/udp # GRE forwarding for g in "${GRES[@]}"; do ufw route allow in on "$EGRESS" out on "$g" ufw route allow in on "$g" out on "$EGRESS" done ufw --force enable ufw status verbose

最终状态

Bash (base) charles@tokyo-2:~$ sudo ufw status Status: active To Action From -- ------ ---- 22/tcp ALLOW Anywhere 80/tcp ALLOW Anywhere 443/tcp ALLOW Anywhere 53/tcp ALLOW Anywhere 53/udp ALLOW Anywhere Anywhere on gre1 ALLOW FWD Anywhere on ens5 Anywhere on ens5 ALLOW FWD Anywhere on gre1 Anywhere on gre2 ALLOW FWD Anywhere on ens5 Anywhere on ens5 ALLOW FWD Anywhere on gre2 (base) charles@tokyo-2:~$ sudo iptables -S ufw-before-input ¦ grep icmp -A ufw-before-input -p icmp -m icmp --icmp-type 3 -j ACCEPT -A ufw-before-input -p icmp -m icmp --icmp-type 11 -j ACCEPT -A ufw-before-input -p icmp -m icmp --icmp-type 12 -j ACCEPT -A ufw-before-input -p icmp -m icmp --icmp-type 8 -j ACCEPT (base) charles@tokyo-2:~$ sudo iptables -S ufw-before-forward ¦ grep icmp -A ufw-before-forward -p icmp -m icmp --icmp-type 3 -j ACCEPT -A ufw-before-forward -p icmp -m icmp --icmp-type 11 -j ACCEPT -A ufw-before-forward -p icmp -m icmp --icmp-type 12 -j ACCEPT -A ufw-before-forward -p icmp -m icmp --icmp-type 8 -j ACCEPT (base) charles@tokyo-2:~$

## 4.3 查看ufw是否启用

Bash sudo ufw status

# 5. 配置NAT

## 5.1 启用 IP 转发与关闭 rp_filter：/etc/sysctl.conf

Plain Text net.ipv4.ip_forward = 1 net.ipv4.conf.all.rp_filter = 0 net.ipv4.conf.default.rp_filter = 0

## 5.2 使之生效：

sudo sysctl -p

# 6. dns解析

## ref：《London使用Tokyo的dns解析binance》